Environment Variables Used By ALL Synopsis/EOH Doors

������������������������������������������������������������<� Environment �>Ŀ

** Port Environment Variable

PORT          Comport Settings, this will automaticly select the use of

the internal UART Routines, it supports the following options:

C<Comport>		Set Comport number to use,

MUST BE THE FIRST!

A<Address>             Base Address of the comport, use a

'$' to specify a hex value.

(Defaults for COM1-4 copied from BIOS)

I<Irq>			Irq Line of the comport.

(Default=COM1,3-Irq4 others Irq3)

B<Baudrate>		Locked Baudrate to use.

(Default=Current)

R<Bytes>		Size of ReceiveBuffer.

(Default=2048)

S<Bytes>		Size of SendBuffer.

(Default=2048)

F<Trigger>		Set Fifo trigger level.

(Default=8)

FOS			Use Fossil like X00, ADF or BNU.

Only the Baudrate will be used.

Example:

SET PORT=C1 A$3F8 I4 B115200 R4096 S4096 F14

** Other Supported Environment Variables

TASK          Current Node/Task, the -N<Node> is not required!

PROMPTS       Path to the Library's with Moving Prompts

RA            Path to Remote Access

DSZLOG        DSZ-Compatible Logfile for the ZModem

COMSPEC       Secondary shell command

TEMP          Temporary Directory for swapping to disk

�������������������������������������������������������������<� End Screen �>��

$$$$S�����S$$��$$$$��$$$$$$$$$$$$S����S$$$�����S$$$$$$S�����$$$��$$$S�����$$$$$

$$' .sS$$S$$$  $$$$  $  sss.`$$',sS$$Ss,`$  $$Ss, `$' .sS$$S$$$  $' .sS$$S$$$$$

$$sssssssss    $$$$  $  $$$$   .$$$$$$$$.`  $$$$$  $sssssssss `  $sssssssss `$$

$.  $$$$$$$ ,  $$$$  $  $$$$   `$$$$$$$$'   $$S�' ,;  $$$$$$$ ,  ;  $$$$$$$ ,$$

$$, `�S$S�',$s,�$$$  $  $$$$  $.`�S$$S�',$  sssS$$$$. `�S$S�',$  $. `�S$S�',$$$

$S$$SssssS��$$$Ssss  $ss$$$$$$$$$SssssS$$$  $$$$$$$$$$SssssS$$$ss$$$SssssS$$$$$

$��$$$$��$  sss$$$$ss$  $$$$$$$$S�����$$$$SS$',$$S����S$$$$$$S����S$$$��$$$$$$$

$  $$$$  $  $$$$  $  $  $$$$  ' .sS$$S$$$$$$'.$',sS$$Ss,`$$',sS$$Ss,`$  sss, `S

$  $$$$  $  $$$$  $  $  $$$$  sssssssss  $$'.$  sssssssss$  $$$$$$$$ `  $$$$  $

$  $$$$  $  $$$$  $  $  $$$$    $$$$$$$ ,$'.$$  $$$$$$$$ ,  $$$$$$$$ ,  $$$$  $

$, �S$$  $, �S$$  $  $, �S$$  s `�S$S�',$'.$$$$.`�S$$$�',$$.`�S$$S�',$  $$$$  $

$$Sssssss$$Sssssss$ss$$Sssssss$$SssssS$$Ss$$$$$$$SssssS$$$$$$SssssS$$$ss$$$$ssS

�������������������������������������������������������������<� End Of Doc �>��